SCHEMATIC ENTRY AND SPICE SIMULATION OF CMOS INVERTER

AIM:

To design and simulate the CMOS inverter and observe the DC and transient responses using cadence tool.

APPARATUS REQUIRED:

- Personal Computer with Linux

- Cadence tool

PROCEDURE

SCHEMATIC ENTRY:

Creating a new library:

- In the library manager, execute File - New library. The new library form appears.

- In the new library form, type ‘my design lib’ in the name section.

- In the field of directory section, verify that the path to the library is set to ~/Database / Cadence- analog – lab –bl3 and click ok.

- In the next ‘technology file for new library form select option attach to an existing tech file and click ok.

- In the ‘attach design library to technology file’ form, select gpdk180 form the cyclic field and click ok.

- After creating a new library you can verify it from the library manager.

- If you right click on the ‘my design lib’ and select properties, you wil find that gpdk180 library is attached as techlib to ‘my design lib’.

Creating a schematic cell view:

- In the CIW or library manager, execute file – new – cell viw.

- Setup the new file form as follows, Do not edit the library path file and the above might be different from the path shown in your form.

- Click ok when done the above setting. A black schematic window for the inverter design appears.

Adding components to schematic:

- In the inverter schematic window, click the instance fixed menu icon to display the add instance form.

- Click on the browse button. This opens up a library browser from which you can select components and the symbol view.

- After you complete the add instance form move your cursor to the schematic window and click left to place a component.

| LIBRARY NAME | CELL NAME |
| --- | --- |
| gpdk180 | PMOS |
| gpdk180 | NMOS |

- This is a table of components for building the inverter schematic.

- After entering components, click cancel in the add instance form or press ESC with your cursor in the schematic window.

Adding pins to schematic:

- Click the pin fixed menu icon in the schematic window. You can execute create pin or press ‘p’.

- Add pin form appears. Type the following in the ADD pin form in the next order leaving space between the pin.

| PIN NAMES | DIRECTION |
| --- | --- |
| Vin,Vdd,Vss | Input |
| Vout | Output |

- Select cancel and then the schematic window enter window file or press the f bind key.

Adding wires to schematic:

- Click the wire (narrow) icon in the schematic window.

- In the schematic window click on a pin of one of your components as the first point for your wiring. A diamond shape appears over the starting point of this wire.

- Follow the prompts at the bottom of design window and click left on the destination point for your wire. A wire is routed between the source and destination points.

- Complete the wiring as shown in the figure and when done wiring press ECS key in the schematic window to cancel wiring.

Saving the design:

Click the check and save icon in the schematic editor window observe CIW output for any errors.

BUILDING THE INVERTER TEST DESIGN:

Creating the inverter test cell view:

- In the CIW or library manager, execute file – new – cell view.

- Setup the newfile as shown below.

- Click ok when done. A blank schematic window for the inverter test design appears.

- Using the components list and properties/ comments in this table build the inverter test schematic.

| LIBRARY NAME | CELL VIEW NAME | PROPERTIES/COMMENTS |
| --- | --- | --- |
| My design lib | Inverter | Symbol |
| Analog lib | Vpulse | Voltage1 = 0, Voltage2 = 1.8, delay Time = 0, Rise time=Fall time=1ns Period=20ns |
| Analog lib | Vdc, gnd | Vdc = 1.8v |

- Add the above components using create – instance or by pressing I.

- Click the wire (narrow) icon and wire your schematic.

- Click create wire name or press c to name the i/p (vsin) and output wires as in below schematic.

- Click on the check and save icon to save the design.

ANALOG SIMULATION WITH SPECTRA:

Starting the simulation environment:

- In the Inverter-test schematic window execute launch – ADEL. The variable virtuoso analog design environment (ADE) simulation window appears.

Choosing a simulator:

- In the simulation window (ADE) execute setup – simulator / directory / host.

- In the choosing simulator form, set the simulator field to specra and click ok.

- In the simulation window (ADE) execute the setup model libraries.

To complete, move the cursor and click ok.

Choosing Analysis:

- Click the choose- Analysis icon in the simulation window (ADE).

- The choosing analysis form appears.

- To Setup the transient analysis.

- In the analysis section select tron.

- Set the stop time as 100ns

- Click at the moderate or enabled button and the bottom and then click apply.

- To set for DC analysis

- In the analysis section select DC.

- Turn on save DC operating point.

- Turn on the component parameters.

- Double click the select Vpulse source or Type V0 (capital V zero).

- Select the DC voltage in the select window parameter and click in the form start and stop voltages are 0 to 1.8.

- Select the enable button and click apply and then click ok.

Selecting output for plotting:

- Execute the o/p’s to be plotted  -select on sschematic in the simulation window.

- Follow the prompt at the bottom. Click on the o/p net vout input vin of the inverter. Press esc with the cursor after selecting.

Running the simulation:

- Execute the simulation Netlist and run in the simulation window to start the simulation on the icon. This will create the netlist as well as run the simulation.

- When the simulation finishes the transient and DC plots automatically will be popped up along with netlist.

SCHEMATIC ENTRY AND SPICE SIMULATION OF

TWO INPUT NAND GATE

CIRCUIT DIAGRAM:

TRANSIENT ANALYSIS

OUTPUT

RESULT:

The Implementation of 2 input NAND gate using CMOS  is constructed and waveforms are verified.

SCHEMATIC ENTRY AND SPICE SIMULATION OF

TWO INPUT NOR GATE

AIM:

To do the schematic entry and to simulate two input NOR gate using CAD tool

CIRCUIT DIAGRAM:

TRANSIENT ANALYSIS

OUTPUT

RESULT:

The Implementation of 2 input NOR gate using CMOS  is constructed and waveforms are verified.